# Motor Grader System Requirements

## Minimum Blade Width

The system shall achieve a minimum blade width of 12 feet (3.7 m) for standard grading operations.

## Six-Way Blade Control

The system shall provide six-way blade control (raise/lower, tilt, angle, side-shift, circle rotation, articulation) operable from the operator station.

## Minimum Turning Radius

The system shall achieve a minimum turning radius of 24 feet (7.3 m) via frame articulation to support maneuvering in confined worksites.

## Minimum Engine Power

The system shall deliver a minimum net engine power of 150 hp (112 kW) to support grading, ripping, and travel operations under full load.

## Operator Visibility

The system shall maintain operator sightlines to the blade, front tires, and rear attachment sufficient to meet applicable visibility standards (e.g., ISO 5006) without obstruction from structural components.

## ROPS/FOPS Cab Protection

The system shall provide a ROPS/FOPS-certified cab meeting applicable rollover and falling-object protection standards (e.g., ISO 3471, ISO 3449).

## Rear Ripper Attachment

The system shall support an optional rear ripper/scarifier attachment capable of penetrating compacted material to a minimum depth of 12 inches (30 cm).

## Transmission Speed Range

The system shall provide a hydrostatic or powershift transmission enabling smooth speed transitions between 0 and 25 mph (40 km/h) without blade control interruption.

## Telematics Integration

The system shall support telematics integration for remote monitoring of engine hours, fuel consumption, fault codes, and location data.

## Maintenance Accessibility

The system shall be serviceable such that scheduled maintenance points (fluid checks, filters, grease points) are accessible without requiring attachment or panel removal.